TITLE: TESTING PROCEDURE FOR STEVAL-MIC008A document

Test procedure for this type of PCB can verify the whole application case

Table of Contents

1.	GENERAL	1

2.	TESTING ENVIRONMENT	1

3.	TESTING EXECUTION	1

3.1	Test 1	1

3.1.1 Preliminary step	1

3.1.2 Test Setup	2

3.1.3 Test	3

4.	Notes for certification	4

GENERAL

| Author: | Stefano Bosco | Project Code: | STEVAL-MIC008A |
| --- | --- | --- | --- |
| Date: | April, 29th 2022 | Board Code: |  |
| Version: | 1.0 |  |  |

TESTING ENVIRONMENT

2.1 Instrumentation and equipment

<list the lab instrumentation and software prerequisites for the testing>

| Item | Type | Features/Description | Reference |
| --- | --- | --- | --- |
| I1 | Software | Audacity | Free download |
| I2 |  |  |  |
| I3 |  |  |  |
| I4 |  |  |  |

2.2 Additional components

<Lists all the additional components needed for testing and specifies what ST will provide to supplier>

| Item | Type | Description | Part Number Reference | Provided by ST (Yes/No) |
| --- | --- | --- | --- | --- |
| C1 | Board | STM32H747I-DISCO |  | YES |
| C2 |  |  |  |  |
| C3 |  |  |  |  |

TESTING EXECUTION

Test 1

Preliminary step

Install Audacity in your PC (https://www.audacityteam.org/download/)

Test Setup

Take STM32H747I-DISCO

Connect STEVAL-MIC008A into STM32I-SDISCO as per next picture

Showing the board on the other side there are 2 micro-USB connectors.

Test

Connect both USB cable to PC

Open Audacity

Verify that the microphone is “STM32Headset”

Start recording pressing red button on Audacity

Put a source waveform with tones, music, speech, or silence close to STEVAL-MIC008A and the Audacity captures the signal coming from microphone filtered inside the microcontroller.

Sop the acquisition. It is possible to hear or to analyze the signal coming from microphone.

Notes for certification

Audacity captures via USB what has been detected to microphone. In case Audacity is not available or STM32 is not recognized, the board STM32H747I-DISCO works properly and the microphones are ever active receiving clock (less than 3MHz) and sending a data out.